# Module -- Diagnostics Manager

# High-Level Description

Handle internal Nexteer Trouble Codes (NTCs) and update the Diagnostic Event Manager (DEM) via a client call to the DEM Interface module which handles the customer specific DTC setting requirements (e.g. DTC masking).

# Figures

## Component Diagram

# Variable Data Dictionary

| Module Inputs | Module Outputs |  |
| --- | --- | --- |
| MEC_Cnt_enum |  | DiagRmpToZeroActive_Cnt_lgc |
| MfgDiagInhibit_Cnt_lgc |  | DiagStsDefTemp_Cnt_lgc |
|  |  | DiagStsHWASbSystmFltPres_Cnt_lgc |
|  |  | DiagStsCtrldDisRmpPres_Cnt_lgc |
|  |  | DiagStsDefVehSpd_Cnt_lgc |
|  |  | DiagRampRate_XpmS_f32 |
|  |  | DiagRampValue_Uls_f32 |
|  |  | DiagStsDefHWAScomExpVal_Cnt_lgc |
|  |  | DiagStsWIRDisable_Cnt_lgc |
|  |  | DiagStsNonRecRmpToZeroFltPres_Cnt_lgc |
|  |  | DiagStsRecRmpToZeroFltPres_Cnt_lgc |

## Module Internal Variables

| Variable Name | Datatype | Resolution | Legal Range (min) | Legal Range (max) | Software Segment {Data Type} |
| --- | --- | --- | --- | --- | --- |
| NTCInfo_Cnt_M_str[512] | NTCInfo_Str | N/A | (see structure definition) | (see structure definition) | DIAGMGR_START_SEC_VAR_CLEARED_UNSPECIFIED |
| RspWhenProcessing_Cnt_M_b32 | uint32 | 1 | FULL | FULL | DIAGMGR_START_SEC_VAR_CLEARED_32 |
| RampDownWhenProcessing_Cnt_M_lgc | boolean | N/A | FALSE | TRUE | DIAGMGR_START_SEC_VAR_CLEARED_UNSPECIFIED |
| ActiveRmpRate_UlspmS_M_f32 | float32 | Single precision float | 0.0001 | 0.5 | DIAGMGR_START_SEC_VAR_CLEARED_32 |
| DiagStatus_Cnt_D_b16 | uint16 | 1 | FULL | FULL | DIAGMGR_START_SEC_VAR_CLEARED_16 |
| NTCInfoQueue_Cnt_M_str[10] | NTCInfoQueue_Str | N/A | (see structure definition) | (see structure definition) | DIAGMGR_START_SEC_VAR_CLEARED_UNSPECIFIED |
| DiagMgrInitialized_Cnt_M_lgc | boolean | N/A | FALSE | TRUE | DIAGMGR_START_SEC_VAR_CLEARED_UNSPECIFIED |
| NTCInfoQueueIndex_Cnt_M_u16 | uint16 | 1 | 0 | 10 | DIAGMGR_START_SEC_VAR_CLEARED_16 |

### User defined typedef definition/declaration

| Typedef Name | Element Name | User Defined Type | Legal Range (min) | Legal Range (max) |
| --- | --- | --- | --- | --- |
| typedef struct { } NTCInfo_Str | Param | uint8 | 0 | 255 |
|  | Status | uint8 | 0 | 255 |
| typedef struct { } FltRsp_Str | Response | uint32 : 24 | FULL | FULL |
|  | DEMEventID | uint32 : 8 | 0 | 255 |
| typedef struct { } NTCInfoQueue_Str | NTC | NTCNumber | 0 | 511 |
|  | Param | uint8 | 0 | 255 |
|  | Status | NxtrDiagMgrStatus | NTC_STATUS_PASSED NTC_STATUS_FAILED NTC_STATUS_PREPASSED NTC_STATUS_PREFAILED |  |

# Constant Data Dictionary

## Calibration Constants

Constant Name

k_FltRspTbl_Cnt_str[]

k_FltRmpRate_UlspmS_f32[]

## Program(fixed) Constants

### Embedded Constants

#### Local

| Constant Name | Resolution | Units | Value |
| --- | --- | --- | --- |
| D_FLTRSPNTCACTIVEBIT_CNT_B32 | N/A | Counts | 0x00800000 |
| D_FLTRSPRECOVERABLEBIT_CNT_B32 | N/A | Counts | 0x00400000 |
| D_FLTRSPHWASBSYSTMFLTBIT_CNT_B32 | N/A | Counts | 0x00200000 |
| D_FLTRSPDEFVEHSPDBIT_CNT_B32 | N/A | Counts | 0x00100000 |
| D_FLTRSPDEFTEMPBIT_CNT_B32 | N/A | Counts | 0x00080000 |
| D_FLTRSPSCOMHWANOTVALIDBIT_CNT_B32 | N/A | Counts | 0x00040000 |
| D_FLTRSPWIRDISABLEBIT_CNT_B32 | N/A | Counts | 0x00008000 |
| D_FLTRSPNTCINHIBITRUNBIT_CNT_B32 | N/A | Counts | 0x00000040 |
| D_FLTRSPNTCINHIBITNOTOPERATEBIT_CNT_B32 | N/A | Counts | 0x00000020 |
| D_FLTRSPRAMPBITS_CNT_B32 | N/A | Counts | 0x0000000F |
| D_TESTFAILEDBIT_CNT_B8 | N/A | Counts | 0x01 |
| D_TESTFAILEDTHISOPCYCLEBIT_CNT_B8 | N/A | Counts | 0x02 |
| D_PENDINGDTCBIT_CNT_B8 | N/A | Counts | 0x04 |
| D_CONFIRMEDDTCBIT_CNT_B8 | N/A | Counts | 0x08 |
| D_TESTNOTCOMPLETESINCELASTCLEARBIT_CNT_B8 | N/A | Counts | 0x10 |
| D_TESTFAILEDSINCELASTCLEARBIT_CNT_B8 | N/A | Counts | 0x20 |
| D_TESTNOTCOMPLETETHISOPCYCLEBIT_CNT_B8 | N/A | Counts | 0x40 |
| D_WARNINGINDICATORBIT_CNT_B8 | N/A | Counts | 0x80 |
| D_RAMPNONE_CNT_U8 | 1 | Counts | 0x0F |
| D_RAMPF2_CNT_U8 | 1 | Counts | 0x0E |
| D_RAMPF1_CNT_U8 | 1 | Counts | 0x0D |
| D_EVTFAILBITS_CNT_B8 | N/A | Counts | D_TESTFAILEDBIT_CNT_B8 ¦ D_TESTFAILEDTHISOPCYCLEBIT_CNT_B8 ¦ D_CONFIRMEDDTCBIT_CNT_B8 |
| D_EVTPASSBITS_CNT_B8 | N/A | Counts | D_TESTFAILEDBIT_CNT_B8 ¦ D_TESTNOTCOMPLETETHISOPCYCLEBIT_CNT_B8 |
| D_RAMP0PCT_ULS_F32 | N/A | Counts | 0.0 |
| D_IGNOREBITSFORSTORAGE_CNT_B8 | N/A | Counts | D_TESTNOTCOMPLETETHISOPCYCLEBIT_CNT_B8 |
| D_DIAGRMPRTLOLMT_ULSPMS_F32 | Single precision float | Uls/mS | 0.0001 |
| D_DIAGRMPRTHILMT_ULSPMS_F32 | Single precision float | Uls/mS | 0.5 |
| D_DIAGSTSNONRECRMPTOZEROBIT_CNT_B16 | N/A | Counts | 0x0001 |
| D_DIAGSTSCTRLDDISRMPBIT_CNT_B16 | N/A | Counts | 0x0002 |
| D_DIAGSTSRECRMPTOZEROBIT_CNT_B16 | N/A | Counts | 0x0004 |
| D_DIAGSTSHWASBSYSTMFLTBIT_CNT_B16 | N/A | Counts | 0x0008 |
| D_DIAGSTSDEFVEHSPDBIT_CNT_B16 | N/A | Counts | 0x0010 |
| D_DIAGSTSDEFTEMPBIT_CNT_B16 | N/A | Counts | 0x0020 |
| D_DIAGSTSSCOMHWANOTVALIDBIT_CNT | N/A | Counts | 0x0040 |
| D_DIAGSTSWIRDISABLEBIT_CNT_B16 | N/A | Counts | 0x0080 |

#### Global

Constant Name

<None>

### Module specific Lookup Tables Constants

| Constant Name | Resolution | Value | Software Segment |
| --- | --- | --- | --- |

# Functions/Macros used by the Sub-Modules

## Library Functions / Macros

The library and functions / Macros that are called by the various sub modules are identified below,

- TableSize_m()

## Data Hiding Functions

- <None>

## Global Functions/Macros Defined by this Module

### Retrieve NTC Failure State

| Function Name | NxtrDiagMgr_GetNTCFailed | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC | NTCNumber | 0 | 511 |
|  | EventFailed | Boolean pointer | FULL | FULL |
| Return Value | (anonymous) | Std_ReturnType | RTE_E_OK |  |

#### Description

### Retrieve NTC Status

| Function Name | NxtrDiagMgr_GetNTCStatus | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC | NTCNumber | 0 | 511 |
|  | Status | uint8 pointer | FULL | FULL |
| Return Value | (anonymous) | Std_ReturnType | RTE_E_OK |  |

#### Description

### Reset Event Status

| Function Name | NxtrDiagMgr_ResetEventStatus | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | None |  |  |  |
| Return Value | (anonymous) | Std_ReturnType | RTE_E_OK |  |

#### Description

### Set Status

| Function Name | NxtrDiagMgr_SetNTCStatus | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC | NTCNumber | 0 | 511 |
|  | Param | UInt8 | 0 | 255 |
|  | Status | NxtrDiagMgrStatus | NTC_STATUS_PASSED NTC_STATUS_FAILED NTC_STATUS_PREPASSED NTC_STATUS_PREFAILED |  |
| Return Value | (anonymous) | Std_ReturnType | RTE_E_OK |  |

#### Description

### Set Operation State

| Function Name | NxtrDiagMgr_SetOperationCycleState | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | OperationCycle | NxtrOpCycle | FULL | FULL |
|  | CycleState | NxtrOpCycleState | FULL | FULL |
| Return Value | (anonymous) | Std_ReturnType | Std_ReturnType |  |

#### Description

### Report NTC Status

| Function Name | NxtrDiagMgr_ReportNTCStatus | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC | NTCNumber | 0 | 511 |
|  | Param | uint8 | 0 | 255 |
|  | Status | NxtrDiagMgrStatus | NTC_STATUS_PASSED NTC_STATUS_FAILED NTC_STATUS_PREPASSED NTC_STATUS_PREFAILED |  |
| Return Value | RetVal | Std_ReturnType | E_OK E_NOT_OK |  |

#### Design Rationale

#### This function is designed to queue up diagnostic faults that occur prior to the diagnostic manager being initialized.  The logic is structured in such a way that there is a limitation that it will only capture the last status being passed to the function for a given NTC if the NTC is called multiple times prior to initialization.  Parameter bits are “OR’d” together for a given NTC.  The function will return E_NOT_OK if the queue is full.

#### Description

## Local Functions/Macros Used by this MDD only

### Set Bits

| Function Name | SetBits_u8 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | const uint8 pointer | 0 | 255 |
|  | BitMask | uint8 | 0 | 255 |
| Return Value | N/A |  |  |  |

#### Description

*Data |= BitMask

### Clear Bits

| Function Name | ClrBits_u8 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | const uint8 pointer | 0 | 255 |
|  | BitMask | uint8 | 0 | 255 |
| Return Value | N/A |  |  |  |

#### Description

*Data &= ~BitMask

### Read Bit

| Function Name | ReadBit_u8 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | uint8 | 0 | 255 |
|  | BitMask | uint8 | 0 | 255 |
| Return Value | (anonymous) | boolean | FULL | FULL |

#### Description

IF 0 = (Data & BitMask) THEN

return FALSE

ELSE

return TRUE

ENDIF

### Set Bits

| Function Name | SetBits_u16 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | const uint16 pointer | FULL | FULL |
|  | BitMask | uint16 | FULL | FULL |
| Return Value | N/A |  |  |  |

#### Description

*Data |= BitMask

### Clear Bits

| Function Name | ClrBits_u16 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | const uint16 pointer | FULL | FULL |
|  | BitMask | uint16 | FULL | FULL |
| Return Value | N/A |  |  |  |

#### Description

*Data &= ~BitMask

### Read Bit

| Function Name | ReadBit_u32 | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | Data | uint32 | 0x000000 | 0xFFFFFF |
|  | BitMask | uint32 | 0x000000 | 0xFFFFFF |
| Return Value | (anonymous) | boolean | FULL | FULL |

#### Description

IF 0 = (Data & BitMask) THEN

return FALSE

ELSE

return TRUE

ENDIF

### Construct the Storage Array

| Function Name | CreateStorageArray | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | OutputStrgPtr | const NTCStrgArray pointer | FULL | FULL |
| Return Value | N/A |  |  |  |

#### Description

### Local Function #n

| Function Name | (Exact name used) | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | (if none, write None) |  |  |  |
|  | (Insert more rows for additional passed arguments) |  |  |  |
| Return Value | (if no value returned, write N/A) |  |  |  |

#### Description

(Place flowchart/design for local function)

# Software Module Implementation

## Runtime Environment (RTE) Initial Values

| Data | Value |
| --- | --- |
| DiagRmpToZeroActive_Cnt_lgc | FALSE |
| DiagStsDefTemp_Cnt_lgc | FALSE |
| DiagStsHWASbSystmFltPres_Cnt_lgc | FALSE |
| DiagStsCtrldDisRmpPres_Cnt_lgc | FALSE |
| DiagStsDefVehSpd_Cnt_lgc | FALSE |
| DiagRampRate_XpmS_f32 | 0 |
| DiagRampValue_Uls_f32 | 0 |
| DiagStsNonRecRmpToZeroFltPres_Cnt_lgc | FALSE |
| DiagStsRecRmpToZeroFltPres_Cnt_lgc | FALSE |
| DiagStsScomHWANotValid_Cnt_lgc | FALSE |
| DiagStsWIRDisable_Cnt_lgc | FALSE |
| MEC_Cnt_enum | ProductionMode |
| MfgDiagInhibit_Cnt_lgc | FALSE |

## Initialization Functions

### Init: DiagMgr_Init1

#### Design Rationale

None

#### Module Internals and Outputs

## Periodic Functions

### Per: DiagMgr_Per1

#### Design Rationale

In this periodic information is collected about all NTCs mapped to given a DTC and the combined information is used to determine whether to update the DTC’s status information:

- DTCs are updated to FAILED if any associated NTC is currently failed regardless of whether all NTCs have been tested.

- DTCs are updated to PASSED if all associated NTCs have been tested and have passed; if any associated NTC has not been tested, the status of the DTC is not updated to PASSED.

- DTCs are not updated to FAILED if an applicable mask is active.

#### Program Flow Start

N/A

#### Store Module Inputs to Local copies

#### Set Appropriate Bits

#### Store Local copy of outputs into Module Outputs

None

#### Program Flow End

N/A

### Per: DiagMgr_Trns1

#### Design Rationale

None

#### Program Flow Start

N/A

#### Store Module Inputs to Local copies

None

#### Construct the Storage Array

CreateStorageArray(Rte_Pim_NTCStrgArray())

Rte_Call_NTCStrgArray_SetRamBlockStatus(TRUE)

#### Store Local copy of outputs into Module Outputs

None

#### Program Flow End

N/A

### Per: DiagMgr_Trns2

#### Design Rationale

#### Program Flow Start

N/A

#### Store Module Inputs to Local copies

None

#### Restart the DEM Operation Cycle

#### Store Local copy of outputs into Module Outputs

None

#### Program Flow End

N/A

## Fault Recovery Functions

None

## Shutdown Functions

None

## Interrupt Functions

None

## Fault Recovery Functions

None

## Shutdown Functions

None

## Interrupt Functions

None

## Serial Communication Functions

### SComm: DiagMgr_SCom_ReadStrgArray

|  |  | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTCStrgArrayPtr | NTCStrgArray pointer | FULL | FULL |
| Return Value | N/A |  |  |  |

#### Design Rationale

None

#### Program Flow Start

N/A

#### Store Module Inputs to Local copies

None

#### Construct the Storage Array

CreateStorageArray(NTCStrgArrayPtr)

#### Store Local copy of outputs into Module Outputs

None

#### Program Flow End

N/A

### SComm: DiagMgr_SCom_GetNTCParamByte

|  |  | Type | Min | Max |
| --- | --- | --- | --- | --- |
| Arguments Passed | NTC | NTCNumber | 0 | 511 |
|  | NTCParamByte | uint8 pointer | FULL | FULL |
| Return Value | N/A |  |  |  |

#### Design Rationale

None

#### Program Flow Start

N/A

#### Store Module Inputs to Local copies

None

#### Construct the Storage Array

*NTCParamByte = NTCInfo_Cnt_M_str[NTC].Param

#### Store Local copy of outputs into Module Outputs

None

#### Program Flow End

N/A

# Execution Requirements

## Execution Sequence of the Module

DiagMgr_Trns1 should run prior to the NVMWriteAll function call that occurs during the shutdown process.

## Execution Rates for sub-modules called by the Scheduler

This table serves as reference for the Scheduler design

| Function Name | Calling Frequency | System State(s) in which the function is called |
| --- | --- | --- |
| DiagMgr_Init1 | Once | COLD_INIT |
| DiagMgr_Per1 | 10 ms | ALL |
| DiagMgr_Trns1 | Triggered | On OFF mode entry |
| DiagMgr_Trns2 | Triggered | On OFF mode exit |

## Execution Requirements for Serial Communication Functions

| Function Name | Sub-Module called by (Serial Comm Function Name) |
| --- | --- |
| DiagMgr_SCom_ReadStrgArray | EPS_DiagSrvcs |
| DiagMgr_SCom_GetNTCParamByte | EPS_DiagSrvcs |

# Memory Map Definition Requirements

## Sub Modules (Functions)

This table identifies the software segments for functions identified in this module.

| Name of Sub Module | Software Segment |
| --- | --- |
| DiagMgr_Init1 |  |
| DiagMgr_Per1 |  |
| DiagMgr_Trns1 |  |
| DiagMgr_Trns2 |  |

## Global and Local Functions

This table identifies the software segments for local functions identified in this module.

| Name of Sub Module | Software Segment |
| --- | --- |
| NxtrDiagMgr_GetNTCFailed |  |
| NxtrDiagMgr_GetNTCStatus |  |
| NxtrDiagMgr_ResetEventStatus |  |
| NxtrDiagMgr_SetNTCStatus |  |
| NxtrDiagMgr_SetOperationCycleState |  |
| SetBits_u8 |  |
| ClrBits_u8 |  |
| ReadBit_u8 |  |
| SetBits_u16 |  |
| ClrBits_u16 |  |
| ReadBit_u32 |  |
| CreateStorageArray |  |

# Known Issues / Limitations With Design

- (Item #1)

# Revision Control Log

| Rev # | Change Description | Date | Author Initials |
| --- | --- | --- | --- |
| 1 | Initial MDD version | 30-Mar-11 | LW |
| 3 | Corrected typo in local function CreateStorageArray. | 04-Apr-11 | YY |
| 4 | Ensure that test-complete bit is accommodated. | 05-Apr-11 | YY |
| 5 | Correct strategy for selecting DTCs to retain in storage-array. | 05-Apr-11 | YY |
| 6 | States and Modes based changes for combining F1 and F2 outputs. | 11-Apr-11 | LWW |
| 7 | Removed Fault masking and replaced Dem direct Event Status access with DemIf Event Status call. | 7-Jun-11 | JJW |
| 8 | Updated for anomaly 3258. Removed unused local functions. | 17-Jun-11 | LWW |
| 9 | Added DiagMgr_Trns2 to restart DEM operation cycle on OFF mode exit (anomaly 2342) CR#4270 | 17-Jun-11 | BDO |
| 10 | Initial component implementation | 13-Jan-12 | LWW |
| 11 | Changed NTCInfo_T to NTCInfo_Str for Data Dictionary compatibility | 27-Jan-12 | OT |
| 15 | Updates to support bit 5, 6 from DTC outline Structure Version 2. Changed the version to 15 to align with the version in synergy | 17-Apr-12 | VK |
| 16 | Updates to mask all faults when in OFF state | 22-Apr-12 | VK |
| 17 | Updates to inihibit warminit bit in all states except operate | 01-May-12 | VK |
| 18 | Anomaly 3313 (MaxRampRate overindexing) | 22-May-12 | OT |
| 19 | Update for new fault response bits | 20-Jun-12 | LWW |
| 20 | Added fault queue to handle faults being called prior to diagnostic manager initialization | 20-Jul-12 | LWW |